System Bootstrap, Version 11.1(10)AA, EARLY DEPLOYMENT RELEASE SOFTWARE (fc1)

Copyright (c) 1997 by cisco Systems, Inc.

C1600 processor with 2048 Kbytes of main memory

program load complete, entry point: 0x4018060, size: 0x1e1568%QUICC_ETHER-1-LOSTCARR: Unit 0, lost carrier. Transceiver problem?program load complete, entry point: 0x8000060, size: 0x2ed9a8

Restricted Rights Legend

Use, duplication, or disclosure by the Government is

subject to restrictions as set forth in subparagraph

(c) of the Commercial Computer Software - Restricted

Rights clause at FAR sec. 52.227-19 and subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer

Software clause at DFARS sec. 252.227-7013.

cisco Systems, Inc.

170 West Tasman Drive

San Jose, California 95134-1706

Cisco Internetwork Operating System Software

IOS (tm) 1600 Software (C1600-Y-L), Version 11.1(12)AA, EARLY DEPLOYMENT RELEASE SOFTWARE (fc1)

Copyright (c) 1986-1997 by cisco Systems, Inc.

Compiled Mon 09-Jun-97 14:20 by krunyan

Image text-base: 0x0801A214, data-base: 0x02005000

cisco 1601 (68360) processor (revision C) with 1536K/512K bytes of memory.

Processor board ID 06977044

Bridging software.

X.25 software, Version 2.0, NET2, BFE and GOSIP compliant.

1 Ethernet/IEEE 802.3 interface.

1 Serial(sync/async) network interface.

System/IO memory with parity disabled

2048K bytes of DRAM onboard

System running from FLASH

8K bytes of non-volatile configuration memory.

4096K bytes of processor board PCMCIA flash (Read ONLY)

--- System Configuration Dialog ---

At any point you may enter a question mark '?' for help.

Use ctrl-c to abort configuration dialog at any prompt.

Default settings are in square brackets '[]'.

Would you like to enter the initial configuration dialog? [yes]:

First, would you like to see the current interface summary? [yes]:

Any interface listed with OK? value "NO" does not have a valid configuration

Interface              IP-Address      OK? Method Status                Protocol

Ethernet0              unassigned      NO  unset  up                    down

Serial0                unassigned      NO  unset  down                  down

Configuring global parameters:

# Enter host name [Router]: cisco

The enable secret is a one-way cryptographic secret used

instead of the enable password when it exists.

# Enter enable secret: cisco

The enable password is used when there is no enable secret

and when using older software and some boot images.

# Enter enable password: cisco

% Please choose a password that is different from the enable secret

# Enter enable password: cisco

# Enter virtual terminal password: cisco

Configure SNMP Network Management? [yes]:

Community string [public]:

Configure IP? [yes]:

Configure IGRP routing? [yes]: no

Configure RIP routing? [no]: no

Configuring interface parameters:

Configuring interface Ethernet0:

Is this interface in use? [yes]:

Configure IP on this interface? [yes]:

# IP address for this interface: 210.64.254.126

# Number of bits in subnet field [0]: 1

% Too few subnet bits

IP address for this interface: 210.64.254.126

Number of bits in subnet field [0]:

Class C network is 210.64.254.0, 0 subnet bits; mask is /24

Configuring interface Serial0:

Is this interface in use? [yes]:

Configure IP on this interface? [yes]:

Configure IP unnumbered on this interface? [no]:

IP address for this interface: 192.168.1.130

Number of bits in subnet field [0]:

Class C network is 192.168.1.0, 0 subnet bits; mask is /24

Note: Configure IP unnumbered on this interface? [no]: yes

The following configuration command script was created:

hostname cisco

enable secret 5 $1$x7vu$FKakU95K92.hbo736NH5R.

enable password cisco

line vty 0 4

password cisco

snmp-server community public

!

ip routing

!

interface Ethernet0

ip address 210.64.254.126 255.255.255.0

!

interface Serial0

ip address 192.168.1.130 255.255.255.0

!

end

# Use this configuration? [yes/no]: y

The enable password you have chosen is the same as your enable secret.

This is not recommended.  Re-enter the enable password.

Building configuration...

[OK]

Use the enabled mode 'configure' command to modify this configuration.

Press RETURN to get started!

%LINK-3-UPDOWN: Interface Ethernet0, changed state to up

%QUICC_ETHER-1-LOSTCARR: Unit 0, lost carrier. Transceiver problem?

%LINK-3-UPDOWN: Interface Serial0, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface Ethernet0, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0, changed state to down

%QUICC_ETHER-1-LOSTCARR: Unit 0, lost carrier. Transceiver problem?

%QUICC_ETHER-1-LOSTCARR: Unit 0, lost carrier. Transceiver problem?

%QUICC_ETHER-1-LOSTCARR: Unit 0, lost carrier. Transceiver problem?

%QUICC_ETHER-1-LOSTCARR: Unit 0, lost carrier. Transceiver problem?

%LINEPROTO-5-UPDOWN: Line protocol on Interface Ethernet0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Ethernet0, changed state to down

%SYS-5-RESTART: System restarted --

Cisco Internetwork Operating System Software

IOS (tm) 1600 Software (C1600-Y-L), Version 11.1(12)AA, EARLY DEPLOYMENT RELEASE SOFTWARE (fc1)

Copyright (c) 1986-1997 by cisco Systems, Inc.

Compiled Mon 09-Jun-97 14:20 by krunyan

cisco>en

Password:

cisco#sh ver

Cisco Internetwork Operating System Software

IOS (tm) 1600 Software (C1600-Y-L), Version 11.1(12)AA, EARLY DEPLOYMENT RELEASE SOFTWARE (fc1)

Copyright (c) 1986-1997 by cisco Systems, Inc.

Compiled Mon 09-Jun-97 14:20 by krunyan

Image text-base: 0x0801A214, data-base: 0x02005000

ROM: System Bootstrap, Version 11.1(10)AA, EARLY DEPLOYMENT RELEASE SOFTWARE (fc1)

ROM: 1600 Software (C1600-BOOT-R), Version 11.1(10)AA, EARLY DEPLOYMENT RELEASE SOFTWARE (fc1)

cisco uptime is 5 minutes

System restarted by power-on

System image file is "flash:c1600-y-l.111-12.AA", booted via flash

cisco 1601 (68360) processor (revision C) with 1536K/512K bytes of memory.

Processor board ID 06977044

Bridging software.

X.25 software, Version 2.0, NET2, BFE and GOSIP compliant.

1 Ethernet/IEEE 802.3 interface.

1 Serial(sync/async) network interface.

System/IO memory with parity disabled

2048K bytes of DRAM onboard

System running from FLASH

8K bytes of non-volatile configuration memory.

4096K bytes of processor board PCMCIA flash (Read ONLY)

Configuration register is 0x2102

cisco#sh conf

Using 386 out of 7506 bytes

!

version 11.1

service udp-small-servers

service tcp-small-servers

!

hostname cisco

!

enable secret 5 $1$x7vu$FKakU95K92.hbo736NH5R.

enable password cisco

!

!

interface Ethernet0

ip address 210.64.254.126 255.255.255.0

!

interface Serial0

ip address 192.168.1.130 255.255.255.0

!

no ip classless

snmp-server community public RO

!

line con 0

line vty 0 4

password cisco

login

!

end

cisco#conf

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

cisco(config)#ip subnet-zero

cisco(config)#int e 0

cisco(config-if)#ip address  210.64.254.126   255.255.255.128

cisco(config-if)#^Z

cisco#

cisco#conf

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

cisco(config)#int s 0

cisco(config-if)#encap ppp

cisco(config-if)#ip route  0.0.0.0  0.0.0.0   s 0

cisco(config)#^Z

cisco#

cisco#write

Building configuration...

[OK]

cisco#sh conf

Using 455 out of 7506 bytes

!

version 11.1

service udp-small-servers

service tcp-small-servers

!

hostname cisco

!

enable secret 5 $1$x7vu$FKakU95K92.hbo736NH5R.

enable password cisco

!

ip subnet-zero

!

interface Ethernet0

ip address 210.64.254.126 255.255.255.128

!

interface Serial0

ip address 192.168.1.130 255.255.255.0

encapsulation ppp

!

no ip classless

ip route 0.0.0.0 0.0.0.0 Serial0

snmp-server community public RO

!

line con 0

line vty 0 4

password cisco

login

!

end

cisco#